PowerPanel® Programming Software README for General Information, Mar 2, 2005

(c) Copyright Autotech Technologies, L.P. 2002

This document provides complementary or late-breaking information to supplement the PowerPanel Programming Software documentation. It also documents changes to the PLC drivers, DLLs, and the PowerPanel Panel Firmware. The most recent version changes are provided first. Previous version changes follow.

README Contents:

Current Revision/Version

PowerPanel Panel Firmware

Revision G.4, Mar 2, 2005

PowerPanel Programming Software

Version 5.4, Mar 2, 2005

Previous Revisions/Versions

# PowerPanel Panel Firmware

Revision G.0, June 15, 2004

Revision F.2, May 13, 2004

Revision F.1, April 16, 2004

Revision F.0, February 19, 2004

Revision E.1, May 30, 2003

Revision E.0, May 9, 2003

Revision D.0, February 10, 2003

Revision C.10, November 11, 2002

Revision C.9, October 16, 2002

Revision C.8, September 30, 2002

Revision C.5, May 28, 2002

Revision C.3, February 20, 2002

Revision C.2, January 22, 2002

Revision C.1, January 7, 2002

Revision B.1, August 8, 2001

Revision B.0, July 17, 2001

PowerPanel Programming Software

Version 5.2, June 15, 2004

Version 5.1, February 19, 2004

Version 4.0, May 22, 2003

Version 3.0, February 10, 2003

Version 2.6, November 11, 2002

Version 2.5, October 1, 2002

Version 2.2, May 28, 2002

Version 2.1, February 20, 2002

Version 2.0, January 7, 2002

Version 1.02-A, August 8, 2001

Version 1.02, July 17, 2001

-----------------------------------------------------------------------------------------------

POWERPANEL  FIRMWARE

Rev G.4	Feb 28, 2004

Problems Fixed

- Fixed a problem drawing line graph objects without frames. The object was drawn 4 pixels too high.

- Fixed a problem in the low-level serial port module that could cause it to return the wrong number of characters.

- Fixed a problem with bar graphs. The interior was not always drawn.

- Fixed a problem with the computer interface reading line graphs. The code was not checking the index for the reading limit correctly. It was reading one past the number of samples. This resulted in a garbage value being returned.

- Fixed a problem with the Monitor Rung Object that resulted in constants sometimes displayed in the pop-up list when a PLC object was touched.

New Features

- Added the XY graph object.

- Added support multi-line alarm messages with time stamps

- Added support for the 6-inch color TFT display

POWERPANEL  PROGRAMMING SOFTWARE

Ver 5.4	Feb 28, 2005

Problems Fixed

- Fixed a problem with expressions that was causing the application to crash

New Features

1.	Added the XY Graph Object

2.	Added the Alarm Preview Object

PLC Driver Updates and additions

(A)

Mitsubishi Computer Link DRIVER revision A

Mitsubishi Computer Link DLL revision A

- New driver supported in version 5.4.

(B)

Universal Ethernet DRIVER revision A

Universal Ethernet DLL revision B

- New driver supported in version 5.4.

(C)

EZAutomation EZPLC DRIVER revision A

EZAutomation EZPLC DLL revision A

- New driver supported in version 5.4.

(D)

Allen-Bradley Data Highway Plus DRIVER revision G

Allen-Bradley Data Highway Plus DLL revision L

A.	Added the ‘ST’ file type.

- Added support for allowing user to add only bit numbers for binary file types. For example, the user can enter B3/200 which would be automatically translated to B3:12/8.

- Made ‘O’ file type R/W from R only.

(E)

Allen-Bradley MicroLogix DF1 (Full Duplex) DRIVER revision D

Allen-Bradley MicroLogix DF1 (Full Duplex) DLL revision G

- Added the ‘ST’ file type.

- Added support for allowing user to add only bit numbers for binary file types. For example, the user can enter B3/200 which would be automatically translated to B3:12/8.

- Made ‘O’ file type R/W from R only.

(F)

Allen-Bradley PLC5 DF1 DRIVER revision C

Allen-Bradley PLC5 DF1 DLL revision G

- Added the ‘ST’ file type.

- Added support for allowing user to add only bit numbers for binary file types. For example, the user can enter B3/200 which would be automatically translated to B3:12/8.

(G)

Allen-Bradley SLC500 DF1 (Full Duplex) DRIVER revision D

Allen-Bradley SLC500 DF1 (Full Duplex) DLL revision G

- Added the ‘ST’ file type.

- Added support for allowing user to add only bit numbers for binary file types. For example, the user can enter B3/200 which would be automatically translated to B3:12/8.

- Added support for ‘O’ and ‘I’ file types.

---------------------------------------------------------------------------------------------------------------

Previous Revisions/Versions

POWERPANEL  FIRMWARE

Rev G.0	June 15, 2004

Problems Fixed

1.	A Downtime Object with less than 2 entries could cause the panel to crash. This has been fixed.

2.	Fixed a problem displaying discrete values as the second data item in Downtime Table objects.

3.	Deleting screens with line graphs while editing online could cause the panel to crash. This has been fixed.

New Features

1.	Added the Multiple PLS object.

2.	Added special screen numbers to display the alarm counts and alarm history. Also added numbers to simulate the buttons on these screens.

3.	Added the time stamp to the Line Graph object.

4.	The scroll buttons on the Line Graph object are blanked when the oldest or newest data is displayed.

Rev F.2	May 13, 2004

Problems Fixed

1.	The F.1 firmware did not recognize the optional RAM card.

New Features

Rev F.1	April 16, 2004

Problems Fixed

1.	The video chip for 6" color and 10" color panels sometimes failed to initialize correctly.

New Features

Rev F.0	February 19, 2004

Problems Fixed

1.	Fixed a problem averaging 2 or more floating point values on a line graph. In earlier revisions this caused incorrect values to be plotted.

2.	Fixed a problem disabling the transmitter on the QUART when the PLC driver sent the disable transmitter command.

New Features

1.	Added non-alphanumeric ASCII keys to the text keypad.

2.	Added support for the CCLink card.

3.	Added the Global Downtime and Downtime Table objects.

4.	Added the elapsed time (hh:mm:ss) format to the Numeric Display object.

5.	Added support for the PowerPLC.

Rev E.1	May 30, 2003

Problems Fixed

1.	Fixed a bug identifying the generic interface card. This caused the touch cells to not work.

2.	If the PLC drivers have an error while writing the initial tag values the writes are aborted. Prior revisions would hang for several minutes if the PLCs were not connected.

New Features

Rev E.0	May 9, 2003

Problems Fixed

1.	Revision D.0 did not read user flash cards created with earlier revisions. This has been fixed

2.	Corrected a typo in the system error message.

3.	Fixed a problem scaling 16-bit unsigned values. If the slope was negative (i.e. as the PLC register increases the display value decreases) then incorrect values were sometimes calculated.

4.	In previous revisions the drivers were not stopped prior to performing a reboot. This sometimes resulted in driver errors during the reboot. The drivers are now stopped before performing a reboot.

5.	Fixed a problem writing string tag values from the computer interface. Previous revisions wrote garbage values.

6.	Fixed a problem saving and writing initial values for string tags.

7.	In previous revisions the “Clear Memory ” command from the program loader could cause a system error if there was a second PLC driver. This has been fixed.

8.	In previous revisions when uploading a user program while in setup mode the COM1 port assignment was changed after the project attributes were received. This caused a time-out error and the program was not loaded correctly. This has been fixed.

9.	Fixed a bug writing string tag values while the program loader is monitoring tags.

New Features

1.	Added expression tags.

2.	The Numeric Entry object was changed to allow separate tags for reading and writing.

3.	Added the Alarm List object.

4.	Added the Screen List Selector object.

5.	Added the Control List Selector object.

6.	Added the Touch PRLS object.

7.	Added handshake flags to the Numeric Entry and Text Entry objects.

8.	Added support for the serial port option card

9.	Added the error tags for the second PLC driver to the project attributes.

10.	When writing a value to a tag the firmware updates the tag value prior to calling the driver’s write routine. This results in the tag having the value even if the driver has an error.

Rev D.0	February 10, 2003

Problems Fixed

1.	Allow the PLC port to control RTS when it is set to RS232 or RS422. Previous revisions only allowed this when set to RS485.

2.	Fixed a problem writing multiple tag values. The exec was removing the entries from the write queue before the driver had finished writing them. This could result in the driver placing incorrect values in the tags. The firmware has been changed to wait until the driver finishes the write before removing entries from the write queue.

3.	Fixed a bug printing alarms. If several alarms are triggered at the same time then the first printed line may have garbage at the beginning of the line.

4.	Alarms that were sent to slaves but not displayed were never cleared from the slave alarm queue. The alarms were continuously sent to slaves even after they were cleared.

New Features

1.	Added initial/retentive tag values.

2.	Added the Global Multi-function object.

3.	Added the Global FIFO data acquisition object.

4.	Added the FIFO table, frequency and Statistics objects.

5.	Added support for dual PLC drivers.

6.	Added the “allow stretching”, “stretch to fit” and “keep aspect ratio” flags to bitmap objects.

7.	Added international characters to the fonts.

8.	Added the port assignments to the project attributes

9.	Allow the user to assign the ports in setup mode.

10.	The multi-state indicator will resend messages to slaves when the embedded data changes.

11.	Added the “number of decimal digits” tag to numeric entry and numeric display objects.

Rev C.10	November 11, 2002

Problems Fixed

1.	The firmware was returning invalid screen titles to the program loader. This could cause the program loader to crash when going online.

New Features

None.

Rev C.9	October 16, 2002

Problems Fixed

1.	On powerup the panel did not always display the correct screen. This has been fixed.

2.	On powerup if an object was in the upper-left corner the panel thought it was pressed. This has been fixed.

3.	Fixed a problem with the alarm history. On power up the clear times were being overwritten with the current time.

New Features

None.

Rev C.8	September 30, 2002

Problems Fixed

1.	The large keypad is not displayed on 320x240 displays.

2.	The computer interface message “Get List of Screen Numbers” (#53) was not returning the correct titles. This has been fixed.

3.	The computer interface message “Read Alarm Count” (#86) returned counts for non-existent alarms. This has been fixed.

4.	Fixed a bug in line graphs with averaged readings that caused an incorrect first sample when the line graph was cleared.

5.	Fixed a problem with the user flash card. The screens in the card were all compared against screen 1 in RAM.

New Features

1.	Added data value “in range”, “equal”, “not equal”, “greater than” and “less than” to alarms.

2.	Line graphs can plot from right to left.

3.	Multi-state bitmaps and indicators allow a mask value for number based bitmap/message selection. The masked value is zero-justified; i.e. a mask of hex C0 allows message numbers 0-3.

4.	Add the interrupt routine for the AnyBus card.

5.	Display the part number for the 15-inch displays.

6.	Increased the initialization time from 2 to 20 seconds.

Rev C.5	May 28, 2002

Problems Fixed

1.	Changed the text area for objects to the full interior area of objects.

2.	The user flash functions have been changed to allow any flash card programmed using exec rev C.4 or later to work with any exec rev C.4 or later. For example if an exec rev D.0 is ever released then it will be able to read a flash card programmed using exec rev C.4. The C.4 exec will also be able to read a flash card programmed using exec rev D.0.

3.	If a momentary pushbutton turns invisible while it is pressed it is treated as if it has been released.

4.	The size of the write queue was changed from 20 values to 40 values.

New Features

1.	Added support for Ethernet communications.

2.	Added large popup keypads for 640x480 displays. A selection for large keypads has been added to system attributes.

3.	Added overlapping objects.

Rev C.3	February 20, 2002

Problems Fixed

1.	The label in the numeric entry popup keypad is trucncated to 20 characters to prevent it from extending out of the keypad window.

New Features

1.	Added the internal switch screen tag to the project attributes.

2	The firmware will support the 1 megabyte RAM card.

3.	The firmware will support the 2 megabyte flash card.

Rev C.2	January 22, 2002

Problems Fixed

1.	Some 6" units would power up with the display  distorted. A timing delay has been added to correct this.

Rev C.1	January 7, 2002

Problems Fixed

1.	Corrected problems that caused the text in objects to be displayed differently between the program loader and the panel. This may result in some objects in earlier user programs not displaying all of the text when used with the new program loader or firmware. To correct this, use the new program loader to stretch the object by 1 or 2 pixels.

2.	Objects that use scaling will now allow the values for point 2 to be less than the values for point 1.

New Features

1.	Added the Multi-state Indicator object.

2	 Added the Multi-state Bitmap object.

3.	Added the Bitmap Button object.

4.	Added the Increment/Decrement Value object.

5.	Added the Draw Frame command.

6.	Added a "Display Frame" selection to all objects. If this is deselected the object is drawn without a frame.

7.	The Recipe object can move values from tag to tag.

8.	Added scaling to the Meter and Bar Graph objects.

9.	Added floating-point support to the Meter, Bar Graph, PID Faceplate and Line Graph objects.

10.	Added password protection for the alarm history and alarm counts to the Project Attributes (Alarm Protection tab). This applies to the Clear buttons on the panel's alarm history and alarm count screens.

11.	Added support for 6-inch units with 16x12 touch cells.

12.	The labels for Numeric Entry objects are displayed on the pop-up keypads. The keypads have been redesigned to prevent accidental pressing of the ENTER key.

13.	Added the Math/Logic object.

14.	Added the Report object.

15.	Added support for PMD/Marquee slaves.

16.	Added a computer interface messages to read alarm counts and line graphs.

17.	The programming port works with RS422.

Rev. B.1	August 8, 2001

Problems Fixed

1.	Panels with a flash card, a memory card and an option card could not program the flash card. This has been fixed.

2.	Some panels had a problem loading the Modbus driver due to the ColdFire register settings. The settings were changed to correct this.

Rev. B.0 July 17, 2001

Initial Release

POWERPANEL  PROGRAMMING SOFTWARE

Ver 5.2	June 15, 2004

Problems Fixed

1.	Corrected printing problems in Alarms and Messages. The Program Loader was printing empty pages and crashing if there are 0 messages or 0 alarms.

2.	Corrected a problem that caused a crash in the Monitor Element Tag list Page while adding a 2nd tag.

New Features

1.	Added the Multiple PRLS Object.

2.	Allow the user to import Touch PRLS tags.

3.	Added the time stamp to the Line Graph object.

4.	Added modem support.

5.	Removed toolbar buttons for Monitor Rung, and Monitor Element.

PLC Driver Updates and additions

Allen-Bradley Data Highway driver revision F

Allen-Bradley Data Highway Plus DLL revision J

A.	The driver was not using the PLC timeout time from the PLC attributes. This has been fixed.

B.	The DLL has been modified to support 32-bit I/O modules.

Ver 5.1	February 19, 2004

Problems Fixed

1.	Selecting "Read Program from Panel and Edit OFF_LINE" for Data Highway Plus projects would always cause the error "M03_7: Unable to store PLC attributes file(*.atr)." during the download. This has been fixed.

New Features

1.	Added the Downtime Global Object. It can be accessed using "Setup/Global Objects/Downtime" menu. Names between FIFO and Downtime will be unique i.e. if a FIFO has been defined with some name, the same name can not be used to create Downtime and vice-versa.

2.	Added the Downtime Table Object. It can be accessed using Objects/Data Acquisition/Downtime, same is available in Data Acquisition toolbar.

3.	Added ONLINE Downtime Reading. In ONLINE mode Downtime data can be read using Panel/Read Downtime from Panel menu.

4.	Added Elapsed Time as a data type in Numeric Display. When the tag's data type is either "UNSIGNED 16" or "UNSIGNED 32" the "Elapsed Time" option appears in Data Type combo box, which when selected grays out "Number of digits " and "Number of Fractional Digits" spin controls.

5.	Following changes in Meter object:

a.	Minimum and Maximum values can be tags.

b.	In Alarms page all the limits can be tags.

6.	Following changes in BarGraph object:

a.	Minimum, Maximum and Midpolar values can be tags as well.

b.	Background color can be transparent. If label is selected the Transparent check box is deselected and grayed out.

7.	"Pick Attributes" and "Apply Attributes" can be accessed using the Edit menu or from the standard toolbar

8.	"Simulate Next State" and "Simulate Previous State" can be accessed using the Edit menu or from the standard toolbar

9.	"Display Tag Name" and "Display Tag Address" can be accessed using the Edit menu or from the standard toolbar

10.	"Zoom Plus", "Zoom Minus" and "Zoom Defualt" can be accessed using the Screen menu or from the standard toolbar

11.	Added support for the embedded PLC. This includes the following:

a.	Touch-PLC panels in Panel list in Project Information dialog box.

b.	Selection of Touch-PLC panel shows a new ladder icon in Project Screen explorer. Clicking on this icon opens the ladder screen.

c.	New submenu items in the following main menus of the Panel Screen (only if Touch-PLC panel is selected):

i.	File: "Copy Rungs"

ii.	Edit: "Go to Rungs" / "Switch to Ladder"

iii.	Objects: "Monitor Rung", "Monitor Element"

iv.	Panel: "Switch to Monitor Mode" / "Switch to Edit Mode"

v.	Setup: Upgrade Firmware (Analog Card)

PLC Driver Updates and additions

1.	Added the Aromat Mewtocol COM driver.

2.	Added the Mitsubishi CCLink driver.

3.	Added the Embedded Touch PLC driver.

4.	Added the Idec Computer Link driver.

5.	DirectLogic Ethernet DLL revision D

A.	The DLL has been modified to suppress error messages when converting a project to Ethernet.

Ver 4.0	May 22, 2003

Requires firmware revision E.0 or later

Problems Fixed

1.	Changing the password names or language names which could corrupt the project file. This has been fixed.

2.	Converting an old project that contained a Dynamic Bitmap object without a frame, could corrupt the project. This has been fixed.

3.	While copying screens if the source project contains the tags in Multi-state Indicator messages, the copy could reference the wrong tags in the destination project.

4.	PowerPanel 3.0 shows default password strings as empty and languages as 1,2,3,…,9 for new projects. This has been fixed.

5.	When the project attributes were edited online the updated tags were not saved. This has been fixed.

6.	When the alarm database was edited online the updated tags were not saved. This has been fixed.

7.	When objects were edited online the updated passwords were not saved. This has been fixed

8.	In revision 3.0 it was possible to select bitmaps that were either higher or wider than the display without resizing the bitmap. The software only resized the bitmap if both the height and width were larger than the display. This has been fixed.

9.	When existing projects were opened the Edit menu selections "TagName as Address" and the "Objects overlapping Warning" were not always set correctly. This has been fixed. When projects prior to 4.0 are opened "TagName as Address" is true and "Objects overlapping Warning" is false. When projects edited with 4.0 or later are opened the selections will be set to the stored values.

New Features

1.	Added support for dual drivers.

2.	Added the Control List Selector object.

3.	Added the Alarm List object.

4.	Added the Touch PRLS object.

5.	Added Notification and Handshake flags to the numeric Entry Object.

6.	Allow different tags for display and data entry in the Numeric Entry object.

7.	Added a “group simulate” for objects. After selecting multiple objects we can see their states by using the Standard toolbar's "Simulate Next State" and "Simulate Previous State" icons or by using the pop-menu that appears upon right clicking the selected objects.

8.	Added the Find & Replace tag name feature to the tag database.

9.	Added the Duplicate Tag feature to the tag database.  Select a tag , right click and select the "Duplicate Tag" option.

10.	Added the Import Alarms function to the setup menu.

11.	Added the ability to copy and paste messages in the message database.

12.	Added expression tags.

PLC Driver Updates and additions

NOTE	All of the driver DLLs have been updated to support dual drivers.

1.	Allen-Bradley MicroLogix DF1 Full Duplex driver revision C

A.	Modified for use as the second PLC driver.

2.	Allen-Bradley SLC 500 DF1 Full Duplex driver revision C

A.	Modified for use as the second PLC driver.

3.	Allen-Bradley SLC 500/MicroLogix AIC driver revision D

A.	Fixed a bug that could cause the driver to report an error when there were no tags on the display.

4.	CTC Binary driver revision B

A.	Modified for use as the second PLC driver.

5.	DirectLogic Ethernet driver revision B

A.	Changed to update the tag value after writing to the ethernet card. This was a problem if the tag was not displayed on the screen, for example if used in a recipe object.

6.	Modicon Modbus RTU driver revision E

A.	Modified for use as the second PLC driver.

Ver 3.0	February 10, 2003

Requires firmware revision D.0 or later

Problems Fixed

1.	Previous revisions show communication progress as a status bar that updates based on elapsed time and not on actual progress. This is changed so that the status bar shows the percentage completed whenever this is possible. This allows the user to judge how much time the operation might require.

2.	In previous revisions if a bitmap size was more than the screen size (640x480 or 320x240) the image could not be selected in a bitmap object. Now the user is asked if he wants the image to be resized to fit in the screen.

3.	In previous revisions the bitmaps were converted from the format required by the program loader to the format required by the panel firmware when the screen was sent to the panel. Now both formats are stored with the user program. This results in a noticeable decrease in the time required to upload user programs that include bitmaps.

4.	Previous revisions did not print multi-line project and screen descriptions correctly. This has been fixed.

New Features

1.	The communication code that was part of the executable in previous revisions is now in the form of a DLL. This DLL is generic and is also used in the PMD Pro software. This DLL is blind to the type of data and was created for use with all of our products.

2.	The maximum size of bitmap objects has been increased from 64K to 512K. This allows larger bitmaps and more bitmaps per multi-state object.

3.	The storage and drawing of bitmaps has been modified to improve performance while editing screens with several bitmaps. This results in a significant improvement over previous revisions when editing screens containing several bitmaps.

4.	Added the “Keep Aspect Ratio” and “Stretch to fit” and "Allow stretching" flags to all of the bitmap objects.

5.	Added changes for tag initial values

6.	Added the Global Multi-function object.

7.	Added the Global FIFO object.

8.	Added the FIFO table, frequency and Statistics objects.

9.	Added the ability to import messages into the message database.

10.	Added the port assignments to the project attributes

11.	The driver .plc files are stored in the application folder. Previous revisions stored them in a subfolder named \drivers.

12.	Added “Close” and “Close All” to the window menu.

13.	Added “Read FIFO From Panel” to the panel menu. This is only active when the panel is online

14.	Allow multiple screens to be selected for deletion.

15.	When the cursor is placed over an object its name and dimensions are displayed on the status bar.

16.	Added the “number of decimal digits” tag to numeric entry and numeric display objects.

PLC Driver Updates and additions

NOTE	All of the driver DLLs have been modified to use a new application interface. These DLLs will not work with earlier versions of the PowerPanel program loader nor will this program loader work with earlier driver DLLs.

1.	Added the Square-D Sy/Max driver.

2.	Added the Texas Instruments Series 5X5 driver.

3.	Allen-Bradley Remote I/O driver revision E

Allen-Bradley Remote I/O DLL revision H

A.	Added support for SLC500 I/O types.

B.	Added a PLC attribute to specify he byte order of strings.

C.	Added an option to switch the byte order of ASCII strings.

3.	DirectLogic K-Sequence driver revision D

A.	Fixed a bug in which tag databases that contained 32-bit or string tags followed by 125 or more tags could cause invalid checksum errors.

3.	Allen-Bradley Remote I/O driver revision E

Allen-Bradley Remote I/O DLL revision H

A.	Added support for SLC500 I/O types.

Ver 2.6	November 11, 2002

Released with firmware C.10

Problems Fixed

1.	Fixed a problem identifying panels with firmware revisions A.1 through B.1. The program loader did not identify the panel type and would not allow the user to upload new programs or firmware.

New Features

PLC Driver Updates and additions

1.	Added the generic Ethernet/IP driver.

2.	Added the generic Profibus DP driver.

3.	Allen-Bradley Remote I/O driver revision B

Allen-Bradley Remote I/O DLL  revision D

A.	Fixed a problem with initializing on a slow network.

B.	Fixed a problem reading string tags when the string was the maximum length. The driver was writing past the end of the tag’s value area.

C.	Added an option to switch the byte order of ASCII strings.

Ver 2.5	October 1, 2002

Released with firmware C.9

Problems Fixed

1.	The "display large key pad on panel" option in project attributes is disabled for smaller (320x240) size panels

2.	The alarm counts are read correctly from the panel. Earlier versions would crash if there were more than 64 alarms.

3.	Fixed a problem saving project attributes, message database and alarm database while editing online.

4.	Fixed a problem saving the screen after monitoring tags while editing online.

5.	Fixed a problem with overlapping objects. If there were two groups of overlapping objects a single end-of-group command was not always included for the first group. This could cause the screen to be corrupted or the application to crash.

6.	Printing the user program caused the program loader to crash. This has been fixed.

7.	Opening a screen program for a 320x240 display with a multi-state indicator or multi-state bitmap object caused the program loader to crash. This has been fixed.

New Features

1.	Added data value “in range”, “equal”, “not equal”, “greater than” and “less than” to alarms.

2.	Line graphs can plot points from right to left.

3.	Multi-state bitmaps and indicators allow a mask value for number based bitmap/message selection. The masked value is zero-justified; i.e. a mask of hex C0 allows message numbers 0-3.

4.	The “change Screen” object accepts a screen number of zero to allow the user to change back to the previously displayed screen.

5.	Added support for 15” TouchPanel.

PLC Driver Updates and additions

1.	Added Modicon Uni Telway driver.

2.	Added the generic DeviceNet driver.

3.	Added CTC 2600-2700 Binary driver.

4.	Allen-Bradley AIC driver revision C

Allen-Bradley AIC DLL revision C

A.	Added support for I (input) and O (output) file types.

B.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

C.	The device name was accidentally dropped when DLL rev B was done. It has been restored.

5.	Allen-Bradley Data Highway Plus DLL revision E

A.	Support for 230K baud was accidentally dropped when revision B was done. It has been restored.

B.	Address ranges were limited to 255. This has been changed to 999 to support the PLC 5.

6.	Allen-Bradley SLC 500 DF1 Full Duplex driver revision B

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

7.	Allen-Bradley SLC 500/MicroLogix DF1 Half Duplex

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

8.	GE Series 90 SNP-X driver revision C

GE Series 90 SNP-X DLL revision D

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

B.	A bug was introduced in DLL revision C that limited the range of Genius Global Data to 255. If a project with tag addresses above 255 was loaded those tags were changed to internal tags. The full range (1-7680) has been restored.

9.	DirectLogic  DirectNet driver revision C

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

10.	DirectLogic  DirectNet 330/340 driver revision D

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

11.	Mitsubishi Direct driver revision B

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

12.	Omron HostLink driver revision F

A.	When a PLC address is used for both a 16-bit tag and a 32-bit tag we would only get the 16-bit tag returned. This was because when processing the 32-bit tag we only received the address of the 16-bit tag.  This gave us the value in the 16-bit tag instead of the 32-bit tag.  This problem has been corrected.

13..	Siemens S7 driver revision C

A.	Writing strings to tags would overwrite other values when the string was longer than the tag. This was fixed by using strncpy in place of strcpy.

Ver 2.2	May 28, 2002

Released with firmware C.5

Problems Fixed

1.	Changed the text area for objects to the full interior area of objects.

2.	When objects are pasted into the screen they are placed 4 pixels down and left of the original object position.

3.	The “same size”, “same height” and “same width” functions use the size of the first object selected instead of the last object selected.

4.	Whenever the user selects the print function he is asked if he wants to save the project.

5.	The name of the math/logic object was changed to multi-function object

New Features

1.	Added the “copy screen” function to the edit menu.

2.	Added copy/paste for the multi-state indicator messages.

3.	Added the “COM Configuration” function to the Panel menu. The user is no longer asked to confirm the COM configuration every time he uses a Panel menu function while offline.

4.	Added the “Display large keypad on panel” check box to the general tab sheet for project attributes.

5.	Added Ethernet support. This includes Ethernet connection for the program loader as well as an Ethernet PLC driver.

6.	Added the ability to import addresses for existing tags. The tags must have the same name and data type.

7.	Added object overlapping

PLC Driver Updates and additions

1.	Modicon Modbus RTU DLL revision D.

A.	The Modbus RTU DLL changed to use a bit number of 0 for discrete tags mapped to bit zero in a register and a bit number of 255 for non-discrete tags. The map entries for these discretes now have “\0” added to the end of the map string. This makes them consistent with tags mapped to other bits in a register.

2.	GE Series 90 SNP-X driver revision B

GE Series 90 SNP-S DLL revision C

A.	The driver and DLL were changed to allow discrete tags to be mapped to a bit in a register. This required the driver to perform a read/modify/write.

3.	Entivity (Think N Do) Modbus DLL revision C

A.	Fixed a bug with long tag descriptions in the Think N Do map file. Tag descriptions of more than 100 characters caused the DLL to crash.

B.	Fixed a problem with string tags in the Think N Do map file. Extra zeros were being added to the address.

4.	DirectLogic K-Sequence driver revision B

DirectLogic K-Sequence DLL revision C

A.	Fixed a problem converting discrete tags from other drivers. The bit numbers for discretes were treated as decimal numbers instead of octal numbers.

B	Fixed a problem when there are more than 123 tags mapped to consecutive PLC addresses. The driver was generating a PLC checksum error.

5	DirectLogic DirectNet driver revision B

DirectLogic DirectNet DLL revision C

A.	Fixed a problem converting discrete tags from other drivers. The bit numbers for discretes were treated as decimal numbers instead of octal numbers.

B	Fixed a problem when there are more than 123 tags mapped to consecutive PLC addresses. The driver was generating a PLC checksum error.

6	DirectLogic DirectNet 330/340 only driver revision C

A	Fixed a problem when there are more than 123 tags mapped to consecutive PLC addresses. The driver was generating a PLC checksum error.

Ver 2.1	February 20, 2002

Released with firmware C.3

Problems Fixed

1.	Lines drawn at the extreme right or bottom edge of the display would cause an object off screen error.

2.	Using bitmap objects under Windows 95 would sometimes cause the program loader to freeze.

New Features

1.	Added the internal switch screen tag to the project attributes.

2.	Added bit masks fo the multi-state bitmap and multi-state indicator objects.

3.	Added support for the Q*2 panels.

Ver 2.0	January 7, 2002

Released with firmware C.2

Problems Fixed

1.	Corrected problems that caused the text in objects to be displayed differently between the program loader and the panel. This may result in some objects in earlier user programs not displaying all of the text when used with the new program loader or firmware. To correct this, use the new program loader to stretch the object by 1 or 2 pixels.

2.	Objects that use scaling will now allow the values for point 2 to be less than the values for point 1.

New Features

1.	The look and feel of the Tag, Alarm and Message Database was changed. They now pop up a dialog box to edit an entry.

2.	Added the ability to export the Tag, Alarm and Message Databases to Excel.

3.	The use can select the default data type for numeric tags.

4.	Added zoom.

5.	Added the ability to resize bitmaps.

6.	Added the ability to import .wmf and .emf files.

7.	Added orthogonal (horizontal/vertical) line drawing. The F8 key is used to enable/disable orthogonal drawing.

8.	Added  Explorer View window to main programming screen that lists all of the screens. The user can double-click on a  screen title to edit that screen.

9.	Added the Multi-state Indicator object.

10.	Added the Multi-state Bitmap object

11.	Added the Bitmap Button object.

12.	Added the Increment/Decrement Value object.

13.	Added the Draw Frame command.

14.	Added a "Display Frame" selection to all objects. If this is deselected the object is drawn without a frame.

15.	The Recipe object can move values from tag to tag.

16.	Added scaling to the Meter and Bar Graph objects.

17.	Added floating-point support to the Meter, Bar Graph, PID Faceplate and Line Graph objects.

18.	Added password protection for the alarm history and alarm counts to the Project Attributes (Alarm Protection tab). This applies to the Clear buttons on the panel's alarm history and alarm count screens.

19.	The Panel Menu items are available while programming offline.

20.	Added the Math/Logic object.

21.	Added the Report object

22.	Added support for PMD/Marquee slaves to alarms.

23.	Added the ability to display the alarm history and counts.

24.	Added the ability to read line graphs and export them to Excel.

25.	Added the ability to monitor and change tag values.

PLC Driver Updates and additions

1.	All DLLs were changed to improve the address sorting. They are all now revision B.

2.	DirectLogic  Modbus driver revision D.

A.	Writing multiple discrete tags did not work when the tags where mapped to bits in consecutive registers. This has been fixed.

3.	Modicon Modbus Driver revision D

Modicon Modbus DLL revision B

A.	Added the byte and word order for numeric and text tags to the attributes.

B.	Writing multiple discrete tags did not work when the tags where mapped to bits in consecutive registers. This has been fixed.

4.	Omron HostLink Driver revision D

A.	The word order used to write 32-bit tags was wrong. It has been changed to LSW-MSW.

B.	The drive would generate error code 12 when reading tags of different sizes mapped to overlapping or consecutive PLC registers. The tag values would be invalid. For example 	a 16-bit tag mapped to address 200 and a 32-bit tag mapped to address 200 would cause this error. The driver was not reading all of the PLC elements. This has been fixed.

5.	Think & Do Driver revision D

A.	Writing multiple discrete tags did not work when the tags where mapped to bits in consecutive registers. This has been fixed.

6.	Added the Control Techniques Unidrive driver.

PowerPanel Programming Version 1.02-A	August 8, 2001

1.	The firmware hex file was updated to rev B.1.

2.	Added the Allen-Bradley Remote I/O driver.

3.	Added the Modicon ModbusPlusdriver.

Ver 1.02	July 17, 2001

Released with firmware B.0

Initial Release